# PERMIT REVIEW SUMMARY

# Permit Writer:	Chappell Ford			 	Date:	April 1, 2026

# Company Name:	Tinsley Resources LLC, Tinsley Pipeline System

# Facility Location:	167 First Street

# Tinsley, MS 39173

# Source Number:	3020-00031

# County: 		Yazoo

# FACILITY DESCRIPTION

# Tinsley Resources LLC owns and operates its Tinsley gathering facility which receives and stores crude oil prior to shipment through the facility’s pipeline to a transfer facility located near Mayersville, MS, or is pumped directly into the Mid Valley pipeline system near Mayersville.  The Tinsley gathering facility receives crude oil via pipeline, with custody transfer taking place at or near the facility boundary. The facility is also equipped with one (1) Lease Automatic Custody Transfer (LACT) unit, so that crude oil may also be received via trucks if the need arises.

# The facility operates two (2) storage tanks, including a 3,360,000 gallon (80,000 bbl) above ground fixed roof steel storage tank (Emission Point AA-001), and a 420,000 gallon (10,000 bbl) above ground internal floating roof storage tank (Emission Point AA-002), also constructed of steel.  Electric-powered pumps are used for the transfer of oil from the storage tanks to the pipeline. The standard industrial code (SIC) for the facility is 1311.

# The potential emissions for the facility are 70.3 tpy of VOC and 4.3 tpy total HAPs, as determined using TANKS ESP software, which utilizes the revised estimation procedures from AP-42, Chapter 7. The facility will therefore be a Synthetic Minor source for VOCs upon issuance of this permit.

| Emission Point | Description |
| --- | --- |
| AA-000 | Tinsley Crude Oil Gathering Facility, including two (2) Storage Tanks, Truck Unloading (LACT) Unit, and Pipeline to Mayersville, MS |
| AA-001 | 3,468,700 Gallon (80,000 Barrel (bbl)) Above Ground Fixed Roof Steel Storage Tank for Crude Oil Vented to the Atmosphere |
| AA-002 | 416,402 Gallon (10,000 bbl) Above Ground Floating Roof Steel Storage Tank for Crude Oil Vented to the Atmosphere |
| AA-003 | Fugitive emissions from tank truck offloading. Crude oil can be offloaded at one (1) Lease Automatic Custody Transfer (LACT) unit, which measure the net volume of liquid hydrocarbons |
| AA-004 | Fugitive emissions from facility equipment leaks (valves, connectors, flanges, etc.) |

# PROJECT/PERMIT ACTION DESCRIPTION

# Tinsley Resources LLC is an existing Air Synthetic Minor Facility who has applied to renew their Air Synthetic Minor Operating Permit (SMOP). Tinsley has the capacity to process as much as 13,500 barrels per day of crude oil causing potential emissions of VOCs to exceed 100 tpy.

# However, Tinsley Resources has a federally enforceable annual throughput limit of 1,533,000 bbl/yr of crude oil  for the fixed roof storage tank (Emission Point AA-001) and 1,022,000 barrels per year of crude oil for the floating roof tank (Emission Point AA-002). Storage of crude oil containing hydrogen sulfide is restricted to the storage tank equipped with a floating roof (Emission Point AA-002).

# The facility is seeking a renewal of their Air Synthetic Minor Operating Permit.

# EMISSION SUMMARY

# The facility does not request any additional modifications along with the renewal of their operating permit. As such, there are no increases or decreases in the facility’s emissions due to this permitting action.

| # Pollutant | # Facility wide PTE, tons/yr1 |
| --- | --- |
| # VOC | # 70.3 |
| # NOX | # 0 |
| # CO | # 0 |
| # PM10 | # 0 |
| # PM2.5 | # 0 |
| # SO2 | # 0 |
| # HAPs | # 4.29 |

# 1 PTE is based on 7,000 barrels per day, the predicted actual throughput for the facility

# APPLICABLE/PROPOSED PERMIT LIMITATIONS

The facility is requesting to retain their current federally enforceable maximum annual throughput limits of

- 4,200 BPD or 1,533,000 BPY for AA-001

- 3,800 BPD or 1,022,000 BPY for AA-002

# NSPS APPLICABILITY

# 40 CFR 60, Subpart Ka - Standards of Performance for Storage Vessels for Petroleum Liquids for Which Construction, Reconstruction, or Modification Commenced After May 18, 1978, and Prior to July 23, 1984

# The 40 CFR 60, Subpart Ka applies to each storage vessel with a storage capacity greater than 151,416 liters (40,000 gallons) that is used to store petroleum liquids for which construction is commenced after May 18, 1978. The facility operates one storage tank (AA-001) that was constructed prior to August 7, 1977 (per application in PER20130001). Therefore, Emission Point AA-001 is not subject to the provisions of Subpart Ka. The facility operates one storage tank (AA-002) that is a floating roof, liquid product storage vessel with a capacity greater than 10,000 barrels (40,000 gallons) that was constructed after May 18, 1978. Therefore, Emission Point AA-002 is subject to the provisions of Subpart Ka.

# 40 CFR 60, Subpart OOOO - Standards of Performance for Crude Oil and Natural Gas Facilities for Which Construction, Modification, or Reconstruction Commenced After August 23, 2011, and on or Before September 18, 2015

# The provisions of Subpart OOOO apply to facilities that commence construction, modification, or reconstruction after August 23, 2011 and on or before September 18, 2015. The facility has not modified or reconstructed the facility. Therefore, the facility is not subject to the provisions of Subpart OOOO.

# MACT APPLICABILITY

# 40 CFR 63, Subpart HH - National Emission Standards for Hazardous Air Pollutants From Oil and Natural Gas Production Facilities

# Subpart HH applies to facilities that are major or area sources of HAPs. Tinsley Resources LLC is a minor source of HAP emissions. Therefore, the facility is not subject to the provisions of Subpart HH.

# 40 CFR 63, Subpart BBBBBB - National Emission Standards for Hazardous Air Pollutants for Source Category: Gasoline Distribution Bulk Terminals, Bulk Plants, and Pipeline Facilities

# Subpart BBBBBB establishes national emission limitations and management practices for hazardous air pollutants (HAP) emitted from area source gasoline distribution bulk terminals, bulk plants, and pipeline facilities. The facility stores crude oil not gasoline. Therefore, the facility is not subject to the provisions of Subpart BBBBBB.

PUBLIC PARTICIPATION

The 30-day public notice period began on July 8, 2026, when the notice was posted on MDEQ’s website and ends August 7, 2026.