# PUBLIC NOTICE PRETREATMENT RATIONALE FOR ISSUANCE

# Anel Corporation

# Carroll County

# Winona, MS

# Water II Branch

# MSP092486

# SIC: 3443

# June 23, 2026

# Permit Engineer: Rusty Parrish

# FACILITY INFORMATION

## Nature of Business: Manufacturer of steel fabricated tanks.

## Process Wastewater Description: Contaminated stormwater runoff from scrap trailers, process wastewater from iron phosphate detergent (iron phosphate coating) and high heated water comingle into an oil/water separator. Sanitary wastewater comingles with the effluent from the oil/water separator before discharging to the Winona POTW.

## Proposed Flowrate: 0.0015 MGD

- Applicable Federal Guidelines: 40 CFR Part 433 – Metal Finishing Point Source Category, specifically 433.17 (PSNS).

## RECEIVING POTW INFORMATION

## POTW Name, AI No., and Permit No.: Winona POTW; AI# 13455; MS0021024

## POTW Treatment Type: Wastewater is treated biologically via oxidation ditch. Disinfection is provided by chlorination. Dechlorination and post-aeration are completed prior to discharge.

- POTW Design Flowrate: 1.2 MGD

- POTW Receiving Stream: Hays Creek

- POTW Receiving Stream 7Q10 (Q7/10): 0.2 cfs = 0.129 MGD

- NOTABLE SAMPLING DATA FROM PERMIT APPLICATION 2P

| Parameter | Long Term Average (mg/L) |
| --- | --- |
| BOD | 143 |
| COD | 581 |
| TSS | 44.6 |
| Ammonia (as N) | 1.41 |
| pH | 7.0 SU |
| Oil & Grease | 38.4 |
| Cadmium | Believed Absent |
| Chromium | Believed Absent |
| Copper | Believed Absent |
| Lead | Believed Absent |
| Nickel | Believed Absent |
| Silver | Believed Absent |
| Zinc | Believed Absent |
| Cyanide | Believed Absent |
| TTO | Believed Absent |

- DEVELOPMENT OF CATEGORICAL LIMITATIONS

40 CFR Part 433 – Metal Finishing Point Source Category; Subpart A – Metal Finishing Subcategory

The term “TTO” shall mean total toxic organics, which is the summation of all quantifiable values greater than 0.01 milligrams per liter for the toxic organics.  A list of the toxic organics is provided in 40 CFR 433.11(e).  For TTO monitoring, the permittee only needs to analyze for those pollutants which would reasonably be expected to be present.  [40 CFR 433.11(e) and 433.12(a)]

The permittee may, in lieu of monitoring for TTO, submit a toxic organic management plan (TOMP) for review and approval.  The TOMP must specify the toxic organic compounds used; the method of disposal used instead of dumping, such as reclamation, contract hauling, or incineration; and procedures for ensuring that toxic organics do not routinely spill or leak into the wastewater.  Upon approval, the permittee may discontinue monitoring for TTO, but the following certification statement must be made in the comment section of every discharge monitoring report submitted:

"Based on my inquiry of the person or persons directly responsible for managing compliance with the permit limitations for total toxic organics (TTO), I certify that, to the best of my knowledge and belief, no dumping of concentrated toxic organics into the wastewaters has occurred since filing of the last discharge monitoring report.  I further certify that this facility is implementing the toxic organic management plan submitted to the MDEQ."  [40 CFR 433.12(a) and (b)]

40 CFR 433.17 – Pretreatment standards for new sources (PSNS)

| Pollutant | Monthly average shall not exceed | Maximum for any 1 day |
| --- | --- | --- |
|  | Milligrams per liter (mg/L) |  |
| Cadmium (T)* | 0.07 | 0.11 |
| Chromium (T) | 1.71 | 2.77 |
| Copper (T) | 2.07 | 3.38 |
| Lead (T) | 0.43 | 0.69 |
| Nickel (T) | 2.38 | 3.98 |
| Silver (T) | 0.24 | 0.43 |
| Zinc (T) | 1.48 | 2.61 |
| Cyanide (T) | 0.65 | 1.20 |
| TTO | N/A | 2.13 |

*The term “T” shall mean total.  [40 CFR 433.11(a)]

In a letter dated August 19, 1998, the EPA states: “For effluent guidelines, for permitting under NPDES, and for other purposes in EPA’s water programs, the terms ‘total metal’ and ‘total recoverable metal’ may be used interchangeably…” .  The metals will be permitted as total recoverable metals.

433.17(c) – Permittee shall not augment the use of process wastewater or otherwise dilute the wastewater as a partial or total substitute for adequate treatment to achieve compliance with the limitations above.

Calculated loads based on the permitted flowrate of 0.007 MGD:

- PROPOSED PERMIT LIMITATIONS

TR = Total Recoverable; BTJ = Best Technical Judgement

Cadmium (TR), Chromium (TR), Copper (TR), Lead (TR), Nickel (TR), Silver (TR), Zinc (TR), and Cyanide (TR) shall be monitored monthly via grab sampling. TTO shall be monitored semi-annually via grab sampling . pH shall be monitored weekly via grab sampling. Flow shall be monitored daily via metering system.

TTO DMR (Discharge Monitoring Report) analyses shall be submitted semi-annually while all other parameters shall be submitted monthly.

Due to lack of available data from Winona POTW and Anel, categorical limits are being established for the initial issuance of this permit.

Within 6 months after permit issuance, the permittee shall submit a complete Pretreatment Form 2P application containing updated data derived from the permitted discharge. Based upon the results of the analyses, the permit may be reopened and reissued to reflect this data.

Allowable Concentration screening for metals will be calculated after sufficient data from the facility and POTW has been submitted.

Changes from 1st Draft:

Description of wastewater to include contaminated stormwater from scrap trailer storage.

After a meeting with the facility via Microsoft Teams on April 14, 2026, it was determined that a metering system would be the best method to measure flow for the process wastewater. All process wastewater is generated using pressure washers and a garden hose which will each contain a flow meter.

APPENDIX A

POTW DATA

The table contains the most current influent and effluent data from the Winona POTW. In the May 2025 MS0021024 NPDES Permit, the POTW is required to submit 12 samples of the parameters in the table above.

ADEQUACY OF TREATMENT

Date: April 6, 2026

Permit Writer:  Rusty Parrish

# Site: Anel Corporation

Pretreatment Permit No. MSP092486

Carroll County

Proposed Hydraulic Loading:  0.050 MGD 	Proposed Flowrate: 0.0015 mgd

The proposed flowrate of 0.0015 MGD is less than the 0.57 MGD available; therefore, the POTW has sufficient hydraulic capacity to accept the proposed discharge.

Note: the POTW average flow values already include the facility’s proposed flow since a discharge has been occurring to the Winona POTW since 2005. The discharge did not require a pretreatment permit until 2025 because of the plant operations/ wastewater effluent type.