# PERMIT RATIONALE FOR REISSUANCE

Fortenberry Slaughter House, Inc.

Pearl River County

June 24, 2026

- FACILITY INFORMATION

Facility Name:  Fortenberry Slaughter House, Inc.

Facility Address:  5739 Highway 43 North Carriere, MS  39426    ,

Water NPDES Permit No.:  MS0037150

EPA ID (FRS Number from EPA ECHO): 110002324324

Standard Industrial Code (SIC):  2011, 2013, and 5147

Permit Writer:  Deanna Rush

EPD Branch:  Water I

- NATURE OF BUSINESS

This permittee has historically operated a slaughterhouse with butchering, custom meet cutting and meat processing including manufacturing sausages; however, in the past two years they have operated as a meat processor and ceased slaughtering. The owners would like to maintain the ability to begin slaughtering if market conditions change. The permit is based on discharging effluent from slaughtering and processing of beef and swine, and from processing deer.  Deer carcasses brought into the facility by hunters are processed October through March.

Standard Industrial Classification (SIC) Codes 2011 (Meat Packing Plant) and 5147 (Meats and Meat Products) apply to this facility. Once slaughtering begins, operations will fall under SIC Code 2011. Both SIC codes require the facility to obtain coverage under an Industrial Stormwater Permit or operate under a No Exposure Certification for Exclusion from Stormwater Permitting.

- EFFLUENT AND RECEIVING STREAM FLOW DATA

Outfall 001:  Effluent from slaughtering and processing at a continuous rate of 360 gallons per day discharges to an unnamed creek then into a tributary of Old River. The 7-day, 10-year minimum stream (7Q10) flow of the unnamed creek is zero.  The creek and tributary are located in the Pearl River Basin.  Sanitary wastewater from a facility restroom is treated in a septic tank system and not part of Outfall 001’s discharge.

- MISSISSIPI’ 2025 LIST OF IMPAIRED WATER BODIES - 303(d) LIST and TOTAL MAXIMUM DAILY LOAD (TMDL)

Mississippi’s 2026 Section 303(d) List does not list Old River. MDEQ listed Fortenberry Slaughterhouse in a Fecal Coliform TMDL for Pearl River, Pearl River Basin, Hancock County, December 2000, ID 500121503. The permit complies with the TMDL since contains fecal coliform and E. coli permit limitations.

- TYPE OF WASTEWATER TREATMENT:

Two stabilization ponds operated in series treat process wastewater.  Each lagoon has a surface area of 4225 square feet. The first lagoon has a depth of 3 to 4 feet with an approximate volume of 94,000 gallons, and the second lagoon has depth of 4-5 feet with an approximate volume 126,000 gallons.  When slaughtering operations are active, the lagoons treat wastewater to removal pollutants in wastewater generated from animal holding pens, slaughtering, cleaning, and sanitizing.

DATA FROM APPLICATION FORM (Form 2C)

| Parameter | Maximum Daily Value |
| --- | --- |
| Biochemical Oxygen Demand 5-day (BOD5) | <65 mg/L |
| Chemical Oxygen Demand | 463 mg/L |
| Total Organic Carbon | 221 mg/L |
| Total Suspended Solids | 1170 mg/L |
| Ammonia (as N) | 27.4 mg/L |
| Total Kjeldahl Nitrogen | 19.7 mg/L |
| Total Nitrogen | 15.9 mg/l |
| Nitrite/Nitrate Nitrogen | 0.29 mg/L |
| Flow | 0 MGD |
| Temperature | 28.1°C |
| pH | 6.9 S.U. |
| Fecal Coliform | >2420 col/100 mL |
| Chlorine, total residual | <0.1 mg/L |
| Oil and grease | 2.4 mg/L |
| Phosphorus | 6.2 mg/L |

Application Note: Data shown above was grabbed from the first lagoon before treatment. Fortenberry’s reported a flowrate of zero during this permitting cycle.

WATER QUALITY LIMITATIONS BASED ON WASTELOAD ALLOCATION

| Parameter | Average |
| --- | --- |
| Flowrate | 360 gallons per day |
| Carbonaceous Biochemical Oxygen Demand 5-day (CBOD5) | 10 mg/L |
| Total Ammonia Nitrogen | 1.37 mg/L |
| Minimum Dissolved Oxygen | 6.0 mg/L |
| E. coli | 126 col/100 ml |
| Chlorine, Total Residual | 0.011 mg/L |
| Total Nitrogen | Report |
| Total Phosphorus | Report |
| pH | 6-9 S.U. |

Wasteload Allocation (WLA) Completed May 7, 2026 by MDEQ Surface Water Division. The WLA established a lower Total Ammonia Nitrogen Limitation of 1.37 mg/L - based on EPA’s guidance in Aquatic Life Ambient Water Quality Criteria for Ammonia Toxicity.

EPA EFFLUENT GUIDELINE LIMITATIONS AND CALCULATIONS

EPA promulgated effluent guidelines for meat slaughterhouses and packinghouse facilities limitations and standards for existing and new meat slaughterhouses and packinghouses (BPT, BAT and NSPS) on February 28, 1974.  Discharges from this operation are subject to Subpart D, High Processing Packinghouse – 40 CFR 432.42 (a)(1) (BPT) and 40 CFR 432.47 (BCT) applicable to facilities that slaughter less than 50 million pounds per year. Packinghouse means a plant that both slaughters animals and subsequently processes carcasses into cured, smoked, canned or other prepared meats products.  High processing packinghouse means a packinghouse, which processes both animals slaughtered at the site and additional carcasses from outside sources. Fortenberry’s processes deer carcasses from outsides sources; therefore, Subpart D is applicable.

40 CFR 432.42 (a) (1) (BPT) for BOD, TSS, fecal coliform, and oil and grease apply to the wastewater.  Categorical limitations for ammonia nitrogen do not apply to this facility because it is an existing facility that slaughters less than 50 million pounds per day. MDEQ developed the permit based on the permittee starting up slaughtering operations with a maximum production rate of 1500 pounds/day (live weight killed LWK).

Categorical Permit Limitations Calculations

| Parameter | EPA Guideline Limitation Factor (Pounds (lbs) of pollutant per 1000 lbs killed (LWK) |  |  | Production Rate in units of 1000 lbs/day | Permit Limitation lbs/day (mg/l) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average |  | Maximum | Multiply EPA Factor by 1000 lbs per day killed | Monthly Average | Daily Maximum |
| BOD5 | 0.24 | 0.48 |  | 1.5 | 0.36 | 0.72 |
| TSS | 0.31 | 0.62 |  | 1.5 | 0.47 | 0.93 |
| Oil and grease | 0.13 | 0.26 |  | 1.5 | 0.2 | 0.39 |
| Fecal Coliform (MPN/100 ml) | N/A | 400 |  | N/A | N/A | 400 |
| pH | 6.0 | 9.0 |  | N/A |  | 9.0 |

TOXICITY SCREENING

The Environmental Permits Division (EPD) did not conduct toxicity screening since the only toxic pollutant reported in the application was chlorine.  For chlorine, monthly average limitation equals the chlorine chronic water quality criteria and daily maximum limitation equals the acute chorine water quality criteria.

ADDITIONAL PERMIT INFORMATION

- Permit Basis, Monitoring and Reporting for limitations in Section XII Table.

A Waste Load Allocation (WLA) recommended a 10 mg/L carbonaceous biochemical oxygen demand (CBOD5) permit limitation.  The current permit has a biochemical oxygen demand (BOD5) limitation of 10 mg/L.  Complying with BOD5 limitation also complies with CBOD5 limitations because BOD5 contains both the nitrogenous biochemical oxygen demand (NBOD) and carbonaceous biochemical oxygen demand (CBOD).  By definition, CBOD5 will be less than BOD5. In addition to the WLA BOD5 limit, a categorical limit of 0.36 lb/day BOD5 is applicable.  The 10 mg/L concentration limit is equivalent to 0.03 lbs/day BOD5; therefore, compliance with the 10 mg/L limit assures compliance with the categorical BOD5 lbs/day limit and is sufficient to meet both the WLA recommendation and the categorical limit.

The permit requires nutrient (phosphorus and nitrogen) monitoring as recommended by the WLA. To determine total nitrogen, the permittee must monitor total Kjeldahl nitrogen (TKN) and nitrate/nitrite, then calculate total nitrogen using the following equation: Total nitrogen =  Total Kjeldahl nitrogen + nitrate/nitrite.

Using technical judgment, MDEQ determined the daily maximum ammonia nitrogen and BOD5 limitations by multiplying their average limitation by a factor of 1.5.

Since E.coli replaced fecal coliform in the state water regulations as the indicator of pathogenic contamination, the permit does not have an average fecal coliform limitation. However, it does contain a maximum fecal coliform limitation because categorical guidelines require maximum fecal coliform limitations.

The permittee must keep daily records of production quantities.

The permittee must report result of sampling and monitoring via Net DMR on an Annual Discharge Monitoring Report (DMR) for Total Nitrogen, Total Kjeldahl Nitrogen, Nitrate+Nitrite Nitrogen and Total Phosphorus.  For all other parameters, the permit must submit a Monthly DMR.

- Draft Permit Changes:

EPA decreased ammonia concentration from 2 mg/L to 1.37 mg/L.

PROPOSED FINAL LIMITATIONS

- Outfall 001 - Average Permit Limitations

| Parameter | Categorical Limitation | Water Quality Limitation | Current Permit Limitation | Proposed Permit Limitation | Sample Frequency | Sample Type | DMR Report Frequency | Basis |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ammonia | N.A. | 2 mg/L | 2 mg/L | 1.37 mg/L | Twice/month | Grab | Monthly | WLA |
| Chlorine | N.A. | 0.011 mg/L | 0.011 mg/L | 0.011 mg/L | Twice/month | Grab | Monthly | WQBEL |
| Flow | None | 360 gpd | 360 gpd | 360 gpd | Twice/month | Instantaneous | Monthly | WLA |
| Oil and grease | 0.2 lbs/day | N.A. | 0.2 lbs/day | 0.2 lbs/day | Twice/month | Grab | Monthly | Categorical |
| BOD5 | 0.36 lbs/day | N. A. | 10 mg/L | 10 mg/L | Twice/month | Grab | Monthly | WLA |
| Dissolved Oxygen | N.A. | 6.0 mg/L minimum | 6.0 mg/L minimum | 6.0 mg/L minimum | Twice/month | Grab | Monthly | WQBEL |
| E. coli (col/100 mL) | N.A. | 126 | None | 126 | Twice/month | Grab | Monthly | WQBEL |
| pH | 6.0 S.U. minimum | 6.0 S.U. minimum | 6.0 S.U. minimum | 6.0 S.U. minimum | Twice/month | Grab | Monthly | WQBEL |
| Production | N.A. | N.A. | Report lbs/day | Report lbs/day | Daily | Measurement | Monthly | Technical Judgment |
| Total Suspended Solids | 0.47 lbs/day | N.A. | 0.47 lbs/day | 0.47 lbs/day | Twice/month | Grab | Monthly | Categorical |
| Total Nitrogen | N.A. | Report lbs/day | Report lbs/day | Report lbs/day | Semiannual | Calculation | Annual | WLA |
| Total Kjeldahl Nitrogen | N.A. | N.A. | Report lbs/day | Report lbs/day | Semiannual | Grab | Annual | Technical Judgment |
| Nitrate+Nitrite | N.A. | N. A. | Report lbs/day | Report lbs/day | Semiannual | Grab | Annual | Technical Judgment |
| Total Phosphorus | N. A. | Report lbs/day | Report lbs/day | Report lbs/day | Semiannual | Grab | Annual | WLA |

WQBEL = Water Quality Based Effluent Limitation, WLA = Waste Load Allocation, TMDL = Total Maximum Daily Load

- Outfall 001 - Maximum Permit Limitations

| Parameter | Categorical Limitation | Water Quality Limitation | Current Permit Limitation | Proposed Permit Limitation | Sample Frequency | Sample Type | DMR Report Frequency | Permit Limit Basis |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ammonia | N.A. | 3 mg/L | 3 mg/L | 2.06 mg/L | Twice/month | Grab | Monthly | Technical Judgment |
| Chlorine | N. A. | 0.019 mg/L | 0.019 mg/L | 0.019 mg/L | Twice/month | Grab | Monthly | WQBEL |
| E. coli | N.A. | 410 col/100 mL | None | 410 col/100 mL | Twice/month | Grab | Monthly | WQBEL |
| Fecal Coliform | N.A. | N.A. | 400 col/100 ml | 400 col/100 mL | Twice/month | Grab | Monthly | Categorical |
| Flow | N.A. | N.A. | Report | Report | Twice/month | Instantaneous | Monthly | Technical Judgment |
| Oil and grease | 0.39 lbs/day | N.A. | 0.39 lbs/day | 0.39 lbs/day | Twice/month | Grab | Monthly | Categorical |
| BOD5 | 0.72 lbs/day | N.A. | 15 mg/L | 15 mg/L | Twice/month | Grab | Monthly | Technical Judgment |
| pH | 9.0 s.u. (max) | 9.0 s.u. (max) | 9.0 s.u. (max) | 9.0 s.u. (max) | Twice/month | Grab | Monthly | WQBEL |
| Production | N.A | N.A. | Report lbs/day | Report lbs/day | Daily | Measurement | Monthly | Technical Judgment |
| Total Suspended Solids | 0.93 lbs/day | N.A. | 0.93 lbs/day | 0.93 lbs/day | Twice/month | Grab | Monthly | Categorical |
| Total Nitrogen | N.A. | Report lbs/day | Report lbs/day | Report lbs/day | Semiannual | Calculation | Annual | WLA |
| Total Kjeldahl Nitrogen | N.A. | N.A. | Report lbs/day | Report lbs/day | Semiannual | Grab | Annual | Technical Judgment |
| Nitrate+Nitrite | N.A. | N.A. | Report lbs/day | Report lbs/day | Semiannual | Grab | Annual | Technical Judgment |
| Total Phosphorus | N. A. | Report lbs/day | Report lbs/day | Report lbs/day | Semiannual | Grab | Annual | WLA |

WQBEL = Water Quality Based Effluent Limitation, WLA = Waste Load Allocation, TMDL = Total Maximum Daily Load