# PRETREATMENT RATIONALE FOR PERMIT ISSUANCE – SECOND DRAFT

WestRock CP, LLC

DeSoto County

Olive Branch, Mississippi

Permit No.: MSP092473

SIC Code: 2653 – Corrugated and Solid Fiber Boxes

May 15, 2026

Permit Writers: Bill Stacy/Carter Cress

# FACILITY INFORMATION

- Nature of Business: Manufacturer of paperboard corrugated packaging.

- Process Wastewater Description: Continuous discharge generated from washing of water-based flexographic printers and gluers

- Proposed Flowrate: Outfall 001: 0.00162 MGD Monthly Average

- Applicable Federal Guidelines: 40 CFR Part 403 – General Pretreatment Regulations for Existing and New Sources of Pollution.

- Process Wastewater Treatment:  No treatment of wastewater occurs prior to introduction into the collection system.

## RECEIVING POTW INFORMATION

- POTW Name; AI No.; and Permit No.: DeSoto County Regional Utility Authority, Metro DeSoto WWTP; 3129; MS0028479

- POTW Treatment Type: 	Activated sludge followed by UV disinfection

- POTW Design Flowrate: 	Phase I: 	0.537 MGD

Phase II:	1.0 MGD

- POTW Receiving Stream: 	Unnamed tributary thence into Nonconnah Creek

- POTW Receiving Stream 7Q10 (Q7/10): 0 MGD

- NOTABLE SAMPLING DATA FROM PERMIT APPLICATION 2P

| Parameter | Daily Max. |
| --- | --- |
| BOD | 314 mg/L |
| COD | --- |
| TSS | 82 mg/L |
| Ammonia (as N) | 9.5 mg/L |
| pH | 7.0 SU |
| Oil & Grease | 14.8 mg/L |
| Cadmium | Non-detect |
| Chromium | Non-detect |
| Copper | 0.0570 mg/L |
| Lead | Non-detect |
| Mercury | Non-detect |
| Nickel | Non-detect |
| Zinc | 0.0694 mg/L |

- EFFLUENT LIMITATION GUIDELINES APPLICABILITY

40 CFR Part 403 – General Pretreatment Regulations for Existing and New Sources of Pollution

The facility is listed as a manufacturer of corrugated and solid fiberboard boxes, however 40 CFR Part 430 – The Pulp, Paper, and Paperboard Point Source Category only applies to facilities manufacturing paperboard from raw materials such as fiber or wood pulp.  Industrial activity conducted at WestRock includes the assembly and printing of paperboard boxes from already manufactured paperboard, therefore, 40 CFR Part 430 is not applicable to the discharge.

No Federal Categorical Effluent Limitations are applicable to the discharge, however, due to non-compliance history at the receiving POTW, the discharge will be evaluated for those parameters for which the POTW’s NPDES permit directly limits.  Loading calculations to the POTW for the applicable parameters are shown below.

(XC) * 8.34 * QAVG = Loading (lb/day)

Where:

XC = Concentration reported in application data (mg/L)

QAVG = Proposed permitted flowrate (MGD)

*Note: for those parameters that were not detected in the effluent sample provided with the application, a value equal to one-half the MQL is used for calculation purposes.

| BOD5 | 314 | * | 8.34 | * | 0.00162 | = | 4.24 | lb/day |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ammonia | 9.5 | * | 8.34 | * | 0.00162 | = | 0.128 | lb/day |
| Cadmium (Cd) | 0.005 | * | 8.34 | * | 0.00162 | = | 0.0000676 | lb/day |
| Copper (Cu) | 0.0570 | * | 8.34 | * | 0.00162 | = | 0.0008 | lb/day |
| Lead (Pb) | 0.003 | * | 8.34 | * | 0.00162 | = | 0.000041 | lb/day |
| Zinc (Zn) | 0.0694 | * | 8.34 | * | 0.00162 | = | 0.00094 | lb/day |
| Mercury (Hg) | 0.00010 | * | 8.34 | * | 0.00162 | = | 0.00000135 | lb/day |

Based on the calculations above and due to a lack of historical data for the industrial discharge, MDEQ proposes that the permittee be required to monitor its discharge for the metals parameters listed above that are directly limited by the POTW’s NPDES permit.  After a period of 180 days, the permittee will be required to resubmit the Form 2P application with the results of the required monitoring.  If evaluation of the additional data reveals a need for numeric limitations or increased monitoring requirements, the permit may be reopened and modified accordingly.  The proposed permit limitations for the initial issuance of this permit are detailed in the following section.

- PROPOSED PERMIT LIMITATIONS

| Parameter | Current Permit Limits (CPL) |  | Proposed Permit Limits (PPL) |  |  |  | Basis |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. (mg/L) | Max. (mg/L) | Avg. (lb/day) | Max. (lb/day) | Avg. (mg/L) | Max. (mg/L) |  |  |
| Cadmium, TR | ----- | ----- | ----- | ----- | Report | Report | BTJ |  |
| Copper, TR | ----- | ----- | ----- | ----- | Report | Report | BTJ |  |
| Lead, TR | ----- | ----- | ----- | ----- | Report | Report | BTJ |  |
| Mercury, TR | ----- | ----- | ----- | ----- | Report | Report | BTJ |  |
| Zinc, TR | ----- | ----- | ----- | ----- | Report | Report | BTJ |  |
| Flow | ----- | ----- | ----- | ----- | Report MGD | Report MGD | BTJ |  |
| pH | ----- | ----- | ----- | ----- | 6.0 min. SU | 9.0 max. SU | BTJ |  |

TR = Total Recoverable; BTJ = Best Technical Judgement

Cadmium (TR), Copper (TR), Lead (TR), Zinc (TR), and Mercury (TR) shall be monitored monthly via composite sampling.  pH shall be monitored weekly via grab sampling.  Flow shall be monitored daily via estimation.

Within 6 months (180 days) after permit issuance, the permittee shall submit a complete Pretreatment Form 2P application containing updated data from the permitted discharge.  Based upon the results of the analysis, the permit may be reopened and modified to reflect this data.

Allowable concentration screening for metals will be calculated after sufficient data from the facility and POTW has been obtained.

A public notice will be required for this permit issuance.

- FACILITY RESPONSE TO FIRST DRAFT PERMIT AND PROPOSED CHANGES FOR SECOND DRAFT PERMIT

The facility submitted an e-mail on April 17, 2026 containing comments about the first draft permit.  The e-mail requested the following changes:

- Change the upper pH limit to 10 or 11 SU;

- Change the monitoring frequency for pH from weekly to monthly;

- Change the sampling procedure for flow from instantaneous sampling to estimation; and

- Change the sampling procedure for the metallic parameters from grab to composite.

### MDEQ provides the following responses to these facility requests:

- The City of Olive Branch’s Sewer Use Ordinance states the following: No person shall discharge or cause to be discharged any of the following described waters or wastes to any public sewers: … Any waters or wastes having a ph lower than 6.0, or greater than 9.0, or having any other corrosive property capable of causing damage or hazard to structures, equipment, and personnel of the sewage works.  Therefore, the permitted pH range shall remain 6.0 – 9.0 SU in the second draft permit;

- Due to the aforementioned denied facility request to expand the permitted pH range above 9.0 SU, the permitted pH monitoring will remain weekly to ensure that the facility maintains the pH of its process wastewater discharge in the permitted range of 6.0 – 9.0 SU;

- The facility made the following statement in the comment e-mail: we do not have a wastewater flow meter, and it would be very difficult to install a flow meter since the sewer manhole is 10’ deep and located in the street.  May we estimate discharge flow each month based on a water balance and water usage?  The request to change the sampling procedure for flow from instantaneous sampling to estimation has been granted; and

- The request to change the sampling procedure for the permitted metallic parameters from grab to composite has been granted.

### Additionally, Project Manager Audrey Lewis, P.E., of Waggoner Engineering, Inc. provided a map in an e-mail received on May 11, 2026.  The map revealed that the wastewater discharges from this facility are received and treated by the Metro DeSoto WWTP.

APPENDIX A

ADEQUACY OF TREATMENT

Date:					April 8, 2026

Permit Writer:				Bill Stacy

Proposed Site:				WestRock CP, LLC

Pretreatment Permit No. MSP092473

Address:				10333 High Point Rd

Olive Branch, MS  38654

DeSoto County

Proposed Hydraulic Loading:	0.00162 MGD

Proposed Organic Loading:	314 mg/L

314 mg/L*8.34*0.00162 MGD = 4.24 lb/day

DeSoto County Regional Utility Authority, Metro DeSoto WWTP

NPDES Permit No. MS0028479, AI No. 3129

Permitted Flow:	0.537 MGD avg. (Phase I)

12 Months Flow:	0.431 MGD max. avg. value (from August 2025 – February 2026 DMRs *View Inc. ceased discharge to Metro WWTF in August 2025)

12 Months BOD5: 	747 lb/day max. avg. influent value (from August 2025 – February 2026 DMRs)

Evaluation of Flow

0.537 MGD permitted – 0.431 MGD actual = 0.106 MGD

0.106 MGD available vs. 0.00162 MGD proposed increase

Evaluation of BOD5

Permitted flow 537,000 gpd/(100 gallons/person/day) = 5,370 population equivalency

5,370*0.17 lb BOD/person/day = 913 lb/day BOD capacity

913 lb/day available – 747 lb/day actual = 166 lb/day

166 lb/day available vs. 4.24 lb/day proposed increase

Proposed Hydraulic Loading:  0.050 MGD 	*Note: Because discharge has been occurring prior to application for a pretreatment permit, the contribution from WestRock is already captured in the existing POTW flow and loading data shown above, but is treated as additional contributory flow for the purposes of this calculation.