# PERMIT REVIEW SUMMARY

# Permit Writer:	Ryan Wigginton 				Date:	May 23, 2026

# Company Name:	Carrier Process Equipment Group, Inc.

# Facility Location:	4526 North Reed Road

# Source Number:	2880-00012

# County: 		Webster

# FACILITY DESCRIPTION

# Carrier Process Equipment Group, Inc., operates an existing manufacturing facility and synthetic minor source in Mathiston, Mississippi.  The facility manufactures industrial air pollution control equipment.  The facility operations fall under SIC Code 3564 and NAICS code 333413 (Industrial and Commercial Fans and Blowers and Air Purification Equipment).

# The facility is comprised of two (2) operations.  The first is fabrication of air pollution control dust collectors.  The second is manufacturing of metal cages utilized inside dust control devices.  Each operates independently; however, each shares some support functions.  Operations at the site include plasma cutting, welding, abrasive blasting, and coating.  The facility also has natural gas-fired operations including space heaters and air make-up units.

# PROJECT/PERMIT ACTION DESCRIPTION

# The facility currently operates under Synthetic Minor Operating Permit (SMOP) 2880-00012.  The facility has submitted an application to renew the SMOP without any proposed changes or revisions.  Changes to this permit are only those to modernize the permit and make conditions consistent with current permitting practices.

# EMISSION SUMMARY

# No decreases or increases of emissions is proposed for this permitting action.

| # Pollutant | # Potential-to-Emit (tpy) |
| --- | --- |
| # Particulate Matter (PM) | # 99.01 |
| # Sulfur Dioxide (SO2) | # 0.01 |
| # Nitrogen Oxides (NOX) | # 2.41 |
| # Carbon Monoxide (CO) | # 2.03 |
| # Volatile Organic Compound (VOC) | # 99.01 |
| # Hazardous Air Pollutant (HAP), Total | # 24.91 |
| # HAP, Individual | # 9.91 |

# 1Emission limit incorporated into the permit for the facility to remain a SMOP and HAP minor source.

# APPLICABLE/PROPOSED PERMIT LIMITATIONS

| Emission Point | Applicable Requirement | Pollutant/ Parameter | Limitation/Standard | Monitoring/Recordkeeping |
| --- | --- | --- | --- | --- |
| AA-000 (Facility-Wide) | 11 Miss. Admin. Code Pt. 2, R. 2.2.B(10)., as established in Synthetic Minor Operating Permit issued June 4, 2015 | VOC | ≤ 99.0 tpy | Monitoring and recording of all HAP/VOC containing materials used at the site |
|  |  | PM10/PM2.5 (filterable + condensable) | ≤ 99.0 tpy | Monitoring and recording of all PM producing materials |
|  |  | PM (filterable only) | ≤ 99.0 tpy |  |
|  |  | HAP | ≤ 9.9 tpy of any individual HAP | Monitoring and recording of all HAP/VOC containing materials used at the site |
|  |  |  | ≤ 24.9 tpy of total HAP |  |
|  | 11 Miss. Admin Code Pt. 2, R. 1.3.F(1). | PM (filterable only) | E = 4.1 × p0.67 | The margin of compliance is large enough to justify no monitoring or recordkeeping |
| AA-005 AA-006 | 11 Miss. Admin Code Pt. 2, R. 1.3.D(1)(a). |  | ≤ 0.6 lb/MMBTU |  |
|  | 11 Miss. Admin Code Pt. 2, R. 1.4.A(1). | SO2 | ≤ 4.8 lb/MMBTU |  |

# NSPS APPLICABILITY

# No equipment or process has a potentially applicable NSPS standard.

# MACT APPLICABILITY

# 40 CFR 63, Subpart XXXXXX (National Emission Standards for Hazardous Air Pollutants for Nine Metal Fabrication and Finishing Source Categories):  this subpart applies to facilities who own or operate an area source engaged in operations in one of nine listed source categories.  The facility could potentially be subject to items 2 and 6 of the list (Fabricated Metal Products and Industrial Machinery and Equipment Finishing Operations, respectively).   However, EPA’s “Nine Metal Fabrication and Finishing Area Source Categories – 40 CFR 63, Subpart XXXXX (6X) NESHAP – Questions and Answers – Revised June 2020,” lists applicable SIC codes for this subpart.  This facility’s operations fall under SIC Code 3564 which is not listed in the EPA document; therefore, this subpart does not apply.

PUBLIC PARTICIPATION

The 30-day public notice period began on August 5, 2026, when the notice was posted on MDEQ’s website and ends September 7, 2026.