# PRETREATMENT RATIONALE FOR REISSUANCE – PUBLIC NOTICE DRAFT

# Weyerhaeuser NR Company, Bruce Facility    	Permit No.: MSP092455

# Calhoun County

# Bruce, Mississippi

# Water II Branch

# SIC Code: 2421

# Permit Writer: Carter Cress

# August 19, 2026

- FACILITY INFORMATION

- Nature of Business: Lumber mill.

- Process Wastewater Description: Continuous discharge of kiln condensate.

- Proposed Flowrate:	Outfall 001: 0.050 MGD Monthly Avg. and Report Daily Max.

D.	Applicable Federal Guidelines: 40 CFR Part 429 – Timber Products Processing Point Source Category; Subpart K – Sawmills and Planing Mills Subcategory.  Specifically, 40 CFR 429.125 – Pretreatment Standards for Existing Sources (PSES).

E.	Process Wastewater Treatment: Oil/water separator thence an aerated lagoon.

- RECEIVING POTW INFORMATION

- POTW Name, AI No., and Permit No.: Bruce POTW, 13040, MS0042129

- POTW Treatment Type: Aerated lagoon system followed by chlorine disinfection.

- POTW Design Flowrate: November – April: 0.50 MGD Maximum Monthly Average

May – October: 0.337 MGD Maximum Monthly Average

- POTW Receiving Stream: Skuna River

- POTW Receiving Stream 7Q10 (Q7/10): 2.48 cfs = 1.60 MGD

POTW Maximum Monthly Average flowrates in Million Gallons per Day (MGD) on year 2025 annual DMR:

| November – April | May – October |
| --- | --- |
| 0.314 | 0.142 |

POTW 12-month Average Flow for year 2025 (QPOTW): 0.228 MGD

- CATEGORICAL REGULATIONS

40 CFR Part 429 – Timber Products Processing Point Source Category; Subpart K – Sawmills and Planing Mills Subcategory.  Specifically, 40 CFR 429.125 – Pretreatment Standards for Existing Sources (PSES).

Any existing source subject to this subpart which introduces process wastewater pollutants into a publicly owned treatment works must comply with 40 CFR Part 403 – General Pretreatment Regulations for Existing and New Sources of Pollution.  [40 CFR 429.125]

IV.	DATA FROM FORM 2-P PERMIT APPLICATION

| Outfall 001 |  |  |
| --- | --- | --- |
| Pollutant | Long-term Average Value | Maximum Daily Value |
| Flow | 0.03 MGD | 0.09 MGD |
| BOD | 23.62 mg/L | 86.0 mg/L |
| Total Suspended Solids | 88.23 mg/L | 330.0 mg/L |
| Oil and Grease | 0.23 mg/L | 5.0 mg/L |
| pH | 6.4 SU (minimum) | 7.3 SU (maximum) |
| COD | --- | 1,340 mg/L |
| Ammonia (as N) | --- | < 0.0539 mg/L |

V.	DATA FROM DISCHARGE MONITORING REPORTS (DMRS)

| Outfall 001 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Year 2023 | Year 2024 | Year 2025 | Year 2026* |
| Largest Average Flow | 0.0436 MGD | 0.0468 MGD | 0.049 MGD | 0.04 MGD |
| Largest Maximum Flow | 0.078 MGD | 0.1172 MGD | 0.087 MGD | 0.087 MGD |
| Largest Average BOD5 | 276 mg/L | 110 mg/L | 140 mg/L | 40.1 mg/L |
| Largest Maximum BOD5 | 510 mg/L | 150 mg/L | 160 mg/L | 78 mg/L |
| Largest Average TSS | 365 mg/L | 295 mg/L | 450 mg/L | 174 mg/L |
| Largest Maximum TSS | 500 mg/L | 410 mg/L | 670 mg/L | 330 mg/L |
| Largest Average O&G | 110.5 mg/L | NODI=B1 | 3 mg/L | NODI=B |
| Largest Maximum O&G | 221 mg/L | NODI=B | 5.4 mg/L | NODI=B |
| Smallest Minimum pH | 6.5 SU | 6.5 SU | 6.4 SU | 6.5 SU |
| Largest Maximum pH | 7.1 SU | 7.4 SU | 7.2 SU | 7.1 SU |

#### *through April

#### 1 NODI=B is an EPA indicator code that means “below detection limit/no detection.”

Please note: bold values indicate violations of current permit limits.  No violations were reported.

VI.	PROPOSED PERMIT LIMITATIONS – OUTFALL 001

| Parameter | Categorical Limits (CAT) |  |  |  | Current Permit Limits (CPL) |  |  |  | Proposed Permit Limits (PPL) |  |  |  | Sampling Frequency | Sample Type | Basis of Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. lb/day | Max. lb/day | Avg. mg/L | Max. mg/L | Avg. lb/day | Max. lb/day | Avg. mg/L | Max. mg/L | Avg. lb/day | Max. lb/day | Avg. mg/L | Max. mg/L |  |  |  |
| BOD5 | --- | --- | --- | --- | Report | Report | Report | Report | Report | Report | Report | Report | Once per Month | Grab | CPL |
| TSS | --- | --- | --- | --- | Report | Report | Report | Report | Report | Report | Report | Report | Once per Month | Grab | CPL |
| Oil and Grease | --- | --- | --- | --- | Report | Report | Report | Report | Report | Report | Report | Report | Once per Month | Grab | CPL |

| Parameter | Categorical Limits (CAT) |  | Current Permit Limits (CPL) |  | Proposed Permit Limits (PPL) |  | Sampling Frequency | Sample Type | Basis of Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Max. | Avg. | Max. | Avg. | Max. |  |  |  |
| Flow | --- | --- | 0.050 MGD | Report MGD | 0.050 MGD | Report MGD | Daily | Totalizer | CPL |
| pH | 5.0 SU (min.) | --- | 5.0 SU (min.) | 9.0 SU | 5.0 SU (min.) | 9.0 SU | Once per Month | Grab | CPL |

The monitoring analyses for BOD5, TSS, oil and grease, flow, and pH shall be submitted on a monthly DMR.

VII.   STATEMENT OF BASIS (narrative)

In a cover letter to the permit application, the facility has requested changes to its monitoring requirements.  The facility requests that the sampling frequency for BOD5, TSS, oil and grease, and pH be reduced from twice per month to once per month.  Additionally, the facility requests that the sample type for BOD5 and TSS be changed from 24-hour composite to grab.

A facility search found two other Weyerhaeuser lumber mills that discharge treated kiln condensate to the local POTWs.  Accordingly, these two facilities possess active water pretreatment permits.  These two facilities are Weyerhaeuser NR Company, McComb Wood Products and Weyerhaeuser NR Company, Philadelphia Facility.  Both of them require once per month sampling of BOD5, TSS, oil and grease, and pH.  Also, both require grab sampling for BOD5 and TSS.  Since no specific concerns were revealed for Weyerhaeuser NR Company, Bruce Facility, the aforementioned requested changes to its monitoring requirements have been granted.

A public notice will be required for this permit reissuance.