# DRAFT PERMIT RATIONALE FOR REISSUANCE

Huey P Stockstill LLC

Pearl River County

Picayune, Mississippi

Water NPDES No. MS0045888

May 6, 2026

- FACILITY INFORMATION

Facility Name:		Huey P Stockstill LLC

Facility Address:	130 Huey Stockstill Road

Picayune, MS  39466

Picayune,

Permit No.:		MS0045888

SIC:			3295

Permit Writer:		Rusty Parrish

EPD Branch:		Water II Branch

- NATURE OF BUSINESS

This facility performs dredge mining of sand and gravel.  The facility also performs ready-mix concrete manufacturing, and these activities are governed by permit number MSG110052.

- EFFLUENT AND RECEIVING STREAM FLOW DATA

The receiving stream is the Pearl River, which is located in the Pearl River Basin and classified as Recreation.  The process wastewater collects in the mine recirculation pond and no discharge occurs under normal operating conditions.  However, discharge via one (1) existing outfall could occur during an extreme rain event that causes the Pearl River to flood into the pond.  At the discharge point, the 7Q10 flowrate of the Pearl River is 1,160 cfs = 750 MGD.  A waste load allocation is not necessary since discharge only occurs during flooding events.

- 303(d) and TOTAL MAXIMUM DAILY LOAD (TMDL) ISSUES

The Pearl River does appear on the Mississippi 2024 List of Impaired Water Bodies for Aquatic Life Use Support due to pH and pathogens. The facility complies with the following applicable TMDLs:

- Fecal coliform – Pearl River – Hancock County – December 2000 – ID: 500121503

- Low pH – Pearl River – Neshoba, Leake, Hinds, Rankin, & Hancock Counties  -December 2000 – ID: 500121501

- Sediment – Pearl River Basin – March 25, 2009 – ID: 50932501

- Total Nitrogen and Total Phosphorus – Pearl River – Pearl River Basin – June 29, 2009

- TYPE OF WASTEWATER TREATMENT

A mine recirculation pond treats process wastewater by sedimentation.

- EPA APPLICABLE CATEGORICAL GUIDELINES

Not Applicable.

DATA FROM APPLICATION FORM 2C

| Parameter | Maximum Daily Discharge |
| --- | --- |
| Biochemical Oxygen Demand (BOD5) | Non-detect |
| Chemical Oxygen Demand (COD) | 82.0 mg/L |
| Total Organic Carbon (TOC) | 7.2 mg/L |
| Total Suspended Solids (TSS) | 81.0 mg/L |
| Ammonia (as N) | Non-detect |
| Oil and Grease | Non-detect |
| pH (minimum) | 6.05 SU |
| Parameter | Maximum Daily Value |

CATEGORICAL GUIDELINE LIMITATIONS CALCULATIONS

Not Applicable.

TOXICITY SCREENING

Not Applicable.

- XI.	PROPOSED FINAL LIMITATIONS

Outfall 001

- Minimum Permit Limitations

| Parameter | Categorical Limitation | Water Quality Limitation | Current Permit Limitation | Proposed Permit Limitation | Basis |
| --- | --- | --- | --- | --- | --- |
| pH | N/A | 6.0 SU | 6.0 SU | 6.0 SU | MSWQS |

- Maximum Permit Limitations:

| Parameter | Categorical Limitation | Water Quality Limitation | Current Permit Limitation | Proposed Permit Limitation | Basis |
| --- | --- | --- | --- | --- | --- |
| pH | N/A | 9.0 SU | 9.0 SU | 9.0 SU | MSWQS |

MSWQS = Mississippi Water Quality Standard.

When operated under normal conditions, there is no process wastewater discharged from the pond.  However, during periods of high water from the Pearl River, the pond can flood, which causes process wastewater to leave the site.  During a calendar year if process wastewater leaves the site only due to flooding, then NODI = 9 shall be reported on the DMR.  If no flooding occurs and no process wastewater leaves the site, then NODI=C shall be reported on the DMR.  [11 Miss. Admin. Code Pt. 6, Ch. 1, Subch. 1.]